WEATHERPROOF MOULDED 
SOUND PROJECTOR

## CAD10/T

### Architects & Engineers Specification

### The unit shall be the PENTON model CAD10/T or equal and approved. The speaker enclosure shall be constructed of a high impact ABS with UV inhibitors. The unit will be weather resistant and fitted with aluminium mesh grille, epoxy coated to match the body. Mounting will be via a hipped aluminium “U” bracket. The speaker assembly shall comprise of a 130mm, polypropylene twin-cone loudspeaker.

A 10watt / 100 volt line, factory fitted transformer shall provide 3dB power taps.

Cable entry shall be via a single cable entry mounted on a removable rear cover.

The loudspeaker shall have a wide dispersion. The cone shall be high a compliance unit with a smooth extended frequency response.

### Rated power 						10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range               	120 ~18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	90dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	2.5/5.8

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

WEATHERPROOF MOULDED 
SOUND PROJECTOR

## CAD10/TC

### Architects & Engineers Specification

### The unit shall be the PENTON model CAD10/TC or equal and approved. The speaker enclosure shall be constructed of a high impact ABS with UV inhibitors. The unit will be weather resistant and fitted with aluminium mesh grille, epoxy coated to match the body. Mounting will be via a hipped aluminium “U” bracket. The speaker assembly shall comprise of a 130mm, polypropylene twin-cone loudspeaker.

A 10watt / 100 volt line, factory fitted transformer shall provide 3dB power taps.

Cable entry shall be via a single cable entry mounted on a removable rear cover.

The loudspeaker shall have a wide dispersion. The cone shall be high a compliance unit with a smooth extended frequency response.

The CAD10/TC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚. A 4-core fire rated cable tail should be added to ensure full compliance.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 						10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range               	120 ~18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	90dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	2.5/5.8

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org