### PRO-SOUND
CEILING LOUDSPEAKER

CCS4/T

Architects and Engineers Specifications

This unit shall be the PENTON model CCS4/T or equal and approved. The loudspeaker baffle shall be a round two part bezel comprising a removable inner metal mesh grille, and ABS plastic chassis. Assembly shall comprise of a sealed rear enclosure, ported at the front for an extended bass response. The unit will be fitted with a single bass/mid-range driver, 115mm in diameter. Complete with a bridged, 25mm metal domed tweeter. The unit shall be fitted with a 30 watt/100 volt line factory mounted transformer. Power taps are to be easily selected via a rotary switch located behind the front grill. Tappings are to be clearly marked on the assembly.

The loudspeaker shall not have any visible fixings.

Connection will be via a 3 way terminal capable of taking up to 2 x 1.5mm² conductors. An integral cable clamp will be provided.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	30w

### Tappings 100v line	30/15/7.5/3.75w
Effective frequency range              	65 ~ 19.5KHz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	86dB                                    
Dispersion at 1k/2kHz                                                 180/160
Axial Q Factor at 1k/2kHz                                    	2.4/4.5

### Colour                                                                        White

### Ceiling Cut-out	168mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org