MOULDED CABINET

LOUDSPEAKER

PBC6/T

# Architects & Engineers Specification

The unit shall be the PENTON model PBC6/T or equal and approved. The enclosure shall be ported with an open back and constructed of high impact ABS plastic, treated with UV inhibitors. The bowed grille will be manufactured from mild steel construction with an epoxy coated finish. The speaker assembly shall comprise of a 160mm diameter bass/mid-range treated coned loudspeaker and shall be fitted with a 6watt / 100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	6w

### Tappings 100v line	6/3/1.5/0.75/0.25w
Effective frequency range	160 ~18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	96dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	3.6/5.8

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

MOULDED CABINET

LOUDSPEAKER

PBC6/TC

# Architects & Engineers Specification

The unit shall be the PENTON model PBC6/TC or equal and approved. The enclosure shall be ported with an open back and constructed of high impact ABS plastic, treated with UV inhibitors. The bowed grille will be manufactured from mild steel construction with an epoxy coated finish. The speaker assembly shall comprise of a 160mm diameter bass/mid-range treated coned loudspeaker and shall be fitted with a 6watt / 100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

The PBC6/TC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	6w

### Tappings 100v line	6/3/1.5/0.75/0.25w
Effective frequency range               	160 ~18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	96dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	3.6/5.8

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org