ROUND METAL 
CEILING LOUDSPEAKER

RCS3/T

# Architects & Engineers Specification

This unit shall be the PENTON model RCS3/T or equal and approved. The loudspeaker baffle shall be a round single construction incorporating a metal mesh grille and an outer pressed steel epoxy coated bezel that has no visible fixings. Installation will be via concealed pre-mounted torsion springs. Assembly shall comprise of an 70mm diameter dual coned  midrange loudspeaker, tropicalized for humid, damp environments and shall be fitted with a 6watt / 100volt line factory fitted transformer with the power taps clearly marked on the assembly.

The field cabling to be connected via a 4 way terminal plastic block.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	6w

### Tappings 100v line	6/4/2w
Effective frequency range               	100 ~ 20Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	87dB                                    
Dispersion at 1k/2kHz                                                 180/180
Axial Q Factor at 1k/2kHz                                    	2.0/2.4

### Colour                                                                        White RAL9016

### Ceiling Cut-out	85mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org