ROUND METAL 
CEILING LOUDSPEAKER

RCS4/T

# Architects & Engineers Specification

This unit shall be the PENTON model RCS4/T or equal and approved. The loudspeaker baffle shall be a round two part construction with an inner metal mesh grille and an outer pressed steel epoxy coated bezel that has no visible fixings. Installation will be via concealed pre-mounted stainless steel torsion springs. Assembly shall comprise of an 80mm diameter dual coned  midrange loudspeaker, tropicalized for humid, damp environments and shall be fitted with a 4watt / 100volt line factory fitted transformer with the power taps clearly marked on the assembly.

The field cabling to be connected via a 3 way terminal plastic block.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	4w

### Tappings 100v line	4/2/1/0.5/0.25w
Effective frequency range	130 ~ 18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	84dB                                    
Dispersion at 1k/2kHz                                                 180/180
Axial Q Factor at 1k/2kHz                                    	1.9/2.2

### Colour                                                                        White RAL9016

### Ceiling Cut-out	119mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

ROUND METAL 
CEILING LOUDSPEAKER

RCS4/FTS

# Architects & Engineers Specification

This unit shall be the PENTON model RCS4/FTS or equal and approved. The loudspeaker baffle shall be a round two part construction with an inner metal mesh grille and an outer pressed steel epoxy coated bezel that has no visible fixings. Installation will be via concealed pre-mounted stainless steel torsion springs. Assembly shall comprise of an 80mm diameter dual coned  midrange loudspeaker, tropicalized for humid, damp environments and shall be fitted with a 4watt / 100volt line factory fitted transformer with the power taps clearly marked on the assembly.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The RCS4/FTS will be suitable for voice alarm applications. In addition to the above specification will be supplied with a steel fire dome capable of withstanding 800˚C. It will be fitted with surface mount ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	4w

### Tappings 100v line	4/2/1/0.5/0.25w
Effective frequency range               	200 ~ 18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	83dB                                    
Dispersion at 1k/2kHz                                                 180/180
Axial Q Factor at 1k/2kHz                                    	1.7/2.2

### Colour                                                                        White RAL9016

### Ceiling Cut-out	119mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org