### ROUND C0-AXIAL METAL CEILING LOUDSPEAKERS

RCS5/TCOAX	      RCS5/FTSCOAX

Architects and Engineers Specifications

This unit shall be the PENTON model RCS5/TCOAX or equal and approved. Assembly shall comprise of a 140mm bass/mid-range coned loudspeaker and a 25mm Mylar dome tweeter complete with 12.5mm moving coil and neodymium magnet. It shall be fitted with a 10watt / 100volt line factory mounted transformer.

The loudspeaker baffle shall be a round two part bezel comprising an inner metal mesh grille, and chassis with integral loudspeaker having no visible fixings. Installation shall be by pre-mounted torsion-springs and cabling via ceramic terminals and a 150˚C thermal fuse. The unit for voice alarm application (RCS5FTSCOAX) must be supplied with a steel fire dome capable of withstanding 600˚C. This loudspeaker must be complaint to BS5839 part 8.

Loudspeaker shall have wide angle dispersion of 180˚.Cone shall be a damped, high compliance type with a smooth extended frequency response over a range of 80Hz ~20kHz. Sensitivity shall be a minimum of 91dB @ 1metre, 1watt test signal bandwidth 100Hz ~10kHz

Transformer shall be 100 volt line with 3dB power taps to be clearly marked on the assembly. Tapped at 10, 5, 2.5 and 1.25 watts.

All units shall be tested in accordance with BS6840 part 5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk  Web: www.penton.org