### ROUND METAL 
CEILING LOUDSPEAKER

RCS8/T

Architects and Engineers Specifications

This unit shall be the PENTON model RCS8/T or equal and approved. The loudspeaker baffle shall be a round two part bezel comprising an inner metal mesh grille, and chassis with integral loudspeaker having no visible fixings. Installation shall be by pre-mounted torsion-springs. The speaker assembly shall comprise of a twin cone loudspeaker and shall be fitted with a 10 watt/100 volt line factory mounted transformer with the power taps clearly marked on the assembly.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range	65 ~ 17Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	94dB                                    
Dispersion at 1k/2kHz                                                 180/110
Axial Q Factor at 1k/2kHz                                    	2.7/6.7

### Colour                                                                        White RAL9016

### Ceiling Cut-out	238mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

### ROUND METAL

### CEILING LOUDSPEAKER

RCS8/FTS

Architects and Engineers Specifications

This unit shall be the PENTON model RCS8/FTS or equal and approved. The loudspeaker baffle shall be a round two part bezel comprising an inner metal mesh grille, and chassis with integral loudspeaker having no visible fixings. Installation shall be by pre-mounted torsion-springs. The speaker assembly shall comprise of a twin cone loudspeaker and shall be fitted with a 10 watt/100 volt line factory mounted transformer with the power taps clearly marked on the assembly.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The RCS8/FTS will be suitable for voice alarm applications. In addition to the above specification will be supplied with a steel fire dome capable of withstanding 800˚C, fitted with 2 x 20mm knock-outs, 3 way ceramic terminals able to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range	150 ~ 17Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	93dB                                    
Dispersion at 1k/2kHz                                                 176/100
Axial Q Factor at 1k/2kHz                                    	3.6/7.8

### Colour                                                                        White RAL9016

### Ceiling Cut-out	246mm

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org