VANDALPROOF METAL

CABINET LOUDSPEAKER

SENTRY4/STC

# Architects & Engineers Specification

This unit shall be the PENTON model SENTRY4/STC or equal and approved. The loudspeaker cabinet shall be a two-part epoxy coated steel fabrication with an inner metal mesh grille covering the drive unit. The speaker assembly shall comprise of an 80mm diameter dual coned midrange loudspeaker and shall be fitted with a 4watt /100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

Cable entry will be via 2 x 20mm cable entries located at the top of the cabinet and via concealed universal mounting holes (for standard UK electrical junction boxes) at the rear.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The SENTRY4/STC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be complaint to BS5839 part 8 & EN60849.

### Rated power 	4w

### Tappings 100v line	4/2/1/0.5w
Effective frequency range               	170 ~20Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	87dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	3.2/3.8

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org